# Incubator for Off Gassing of Materials

### Supplies

| Hardware | Price $ | Amount |
| --- | --- | --- |
| Air Heater/ Fan (PTC Car Heater 12V 100W) | 23.73 | 1 |
| Adjustable Power Supply | 15.99 | 3 |
| Coleman Cooler | 39.99 | 1 |
| PTFE Film Adhesive | 58.27/ foot | ~20ft |
| BOJACK DS18B20 W1ThermSensor | 8.99 | 1 |
| 4.7K Ohm Resistors 1/2W | 5.99 | 1 |
| 2 Channel 5V Relay | 6.79 | 1 |
| Breadboard Jumper Ribbon Cables | 5.97 | 1 |
| Raspberry Pi 5 | 159.00 | 1 |
| Portable Monitor | 84.99 | 1 |
| Logitech Keyboard | 129.99 | 1 |
| Logitech Mouse | 13.40 | 1 |
| GPIO Breakout Kit for Raspberry Pi | 9.99 | 1 |
| 1/4" Tubing OD to 1/4 NPT Male fitting | 4.90 | 2 |
| 1/2 NPT Male to 1/4 NPT Female Adapter | 3.88 | 2 |
| 1/2 NPT Female Through-Wall Fitting | 35.44 | 2 |
| 1-3/8" Diameter Hole Saw | 16.27 | 1 |

Heater-control-w1thermsensor.py provides a solution for controlling a relay-based heater/fan system using a temperature reading from the w1thermsensor. The system is designed to maintain a desired temperature range by turning the heater on or based on the predefined threshold while Zero air runs through the cooler.

### Features:

- Temperature monitoring: read_temperature reads temperature from W1ThermoSensor connected to specified GPIO pin, log_temperature records temperature as a function of time to a CSV file

- Relay control: control_relay activates or deactivates a relay connected to specified GPIO pin based on a specified temperature range

- Configurable threshold: bariables TEMP_UPPER_LIMIT and TEMP_LOWER_LIMIT allows for easy adjustment for the temperature threshold at which the relay could be switched

- Error handling: Includes a retry mechanism and SensorNotReadyError pass statement for W1Thermosensor readings to improve reliability

- GPIO cleanup: Ensure proper cleanup of GPIO settings upon program termination

### Run code

- All Code was written and run through VSCode, where the file can be downloaded as a jupyter notebook

Or

Sudo python5 heater-control-w1thermsensor.py

### Live Setups:

Figure 1 Live Setup GPIO Setup

Figure 2 Relay Controller Heater (H), Fan (F)

Figure 3 Incubator Setup as of 06/23/2025

### Wiring Diagram

Figure 4 Wiring Diagram using W1ThermSensor

### Incubator Diagram

Figure 5 Incubator Diagram including measurements

### Through-wall Fittings Diagram

Figure 6 Inlet/Outlet Breakdown for Through-wall Fittings